Real Time Operating System

Name – Haygriv Bansal

Roll No – 2410994538

Group – X2

Course Coordinator – DR Amit Kumar

Course Code – 24ECE4202

Problem Statement:

The given problem statement was Industrial Emergency Stop Reporter , we have configure onboard Pins to display message on external USART software serial port utility . In this problem statement solution we configured onboard Pin PA5 , PC13 , PA0 , PA2 , PA3 and

HSE AND BSE as BYPASS CLOCK Source.

Block Diagram:

Pin Mapping:

| Function | STM32 Pin | Mode Configuration | Description |
| --- | --- | --- | --- |
| Emergency Button | PC13 | GPIO Input with Pull-Up | Detects button press (Active LOW) |
| RED LED | PA5 | GPIO Output Push-Pull | Indicates emergency condition |
| UART TX | PA2 | USART2_TX (AF mode) | Transmit messages to PC |
| UART RX | PA3 | USART2_RX (AF mode) | Receive "RESET" command |
| Timer (Debounce) | TIM2 | Basic Timer (200 ms delay) | Software debounce control |

System Configuration:

Step 1:

Configure Pin PA5 as GPIO output pin and Pin PC13 as GPIO input pin

Step 2:

Go to system core and select PC13 pin and set GPIO mode as Pull up mode

Step 3:

Go to system core go to RCC and enable HSE and BSE as BYPASS Clock Source

Step 4:

Go to Timers and select TIM2 , enable Channel 1 as PWM Generation CH1 and clock source as internal clock , then give values to Prescaler = 84 -1 and ARR = 1000 – 1 .

Step 5:

Go to Conectivity and select USART2 and enable its mode as Asynchronous and give the value of

Baud Rate = 11520

Word Length = 8bits

Parity = None

Stop Bits = 1

Step 6:

Go to clock configuration and edit the value of HCKL clock to 84 and press enter and generate code.

Source Code:

Write this part under Private Variables.

uint8_t rx_buffer[10];

uint8_t alert_msg[] = "ALERT: stoped\r\n";

uint8_t normal_msg[] = "SYSTEM_NORMAL\r\n";

write this code in While 1 loop.

Observation:

- When the system was powered ON, the Red LED remained OFF, indicating normal condition.

- When the emergency push button was pressed, the Red LED turned ON after a short delay (around 200 ms).

- The message "ALERT: Conveyor_Stopped" was successfully transmitted to the serial monitor through UART.

- The 200 ms debounce delay prevented false triggering due to button bouncing.

- When the command "RESET" was sent through the serial terminal, the Red LED turned OFF.

- The message "SYSTEM_NORMAL" was transmitted after receiving the reset command.

- The system remained responsive to UART communication even during button press handling.

- No unexpected behavior or multiple triggers were observed during testing.

Result :

The system correctly detected the emergency button press with 200 ms debounce and activated the Red LED. It transmitted the message "ALERT: Conveyor_Stopped" through UART during emergency condition.

Upon receiving the "RESET" command, the system turned OFF the Red LED and transmitted "SYSTEM_NORMAL", restoring normal operation.

System Designed ;