AIM- To develop a basic FreeRTOS-based project on STM32F446RE in STM32CubeIDE and validate LED blinking using a single RTOS task.

APPARATUS- STM32 Nucleo-F446RE development board, USB Type-A to Mini-B cable, STM32 CUBE IDE, STM32 CubeMX.

THEORY

A Real-Time Operating System (RTOS) such as FreeRTOS replaces the conventional super-loop architecture with a preemptive scheduler that manages multiple independent tasks, each having its own stack, priority, and execution state. In this experiment STM32F446RE uses CMSIS-RTOS v2, a single FreeRTOS task (`Task1_function`) is created via `osThreadNew()` to periodically toggle the onboard LED (PA5); the task calls `osDelay(500)` to block itself for 500 ms after each toggle, yielding CPU control back to the scheduler and creating a precise 500 ms blink period while keeping the processor available for future tasks. Standard `printf` statements within the task are redirected through a custom `_write()` function to the SWV ITM Data Console via ITM stimulus port 0, enabling non-intrusive real-time observation of task execution alongside the visible LED blinking, thus demonstrating both RTOS task management and embedded debugging techniques fundamental to Cortex-M4 development.

PROCEDURE

- Open CubeMX and Click on ACCESS TO MCU SELECTOR.

- Write and select the microcontroller as you want to use and then click start the project. Select commercial part number STM32L446RE.

- In system Core select GPIO and right click on pin PA5 to make it as GPIO output pin as shown below as LED (Inbuilt LED).

- Clock Configuration: Configure clock as shown below to get maximum internal clock frequency of 84MHz.

- SYS Mode: Click on SYS mode and select debug as Trace Asynchronous Sw, and Timebase source to TIM6.

- FreeRTOS Configuration: Click on Middleware and Software Pack, then select FreeRTOS software pack, which is already available with STMCube Mx. Select interface as CMSIS_V2.

- Click on Task & Queue Tab of configuration setting and create a Task_1 as shown below:

- Write Project name and select IDE.

- Click on generate code.

- Click on open project.

- Click ‘OK’ on the generated popup showing “successfully imported the project on the workspace” and the project will be successfully added to the cube IDE.

- Open the main source code on Cube IDE by clicking on main.c

- Click on project properties, go to C/C++ Build  settings  check box for printf and scanf settings.

- Now click on Debug icon and configure debugger for SWV ITM Trace

- Click on Debugger configuration, select debugger console, and click on ST-LINK S/N and then scan, as we attach a STM32F446RE development board, it will automatically pick up ST-LINK id.

- Next, select the Serial Wire Viewer (SWV) and check box on enable button and make sure to pass on the right core clock frequency which is set-up at per point 4 here i.e. 84MHz. Then click on Apply button and close this window.

- After configuration write following instructions in the main.c file and compile the program and ensure there will be zero error after compiling.

/* USER CODE BEGIN Includes */

#include <stdio.h>

/* USER CODE END Includes */

Write the function to enable SWV ITM Console to trace the output of Task_1

/* USER CODE BEGIN 0 */

// Retarget fwrite to ITM

int _write(int file, char *ptr, int len) {

for (int i = 0; i < len; i++) {

ITM_SendChar(*ptr++);

}

return len;

}

/* USER CODE END 0 */

Edit the Task_1 function as given below to toggle the inbuilt LED for 500ms delay.

* USER CODE END Header_Task1_function */

void Task1_function(void *argument)

{

/* USER CODE BEGIN 5 */

/* Infinite loop */

for(;;)

{

HAL_GPIO_TogglePin(LED_GPIO_Port, LED_Pin);

printf("Task_1 Executing for LED Toggle \n");

osDelay(500);

}

/* USER CODE END 5 */

- Now, click on Window tab, select show view other  select SWV ITM Data Console.

- Now click on Debug button, start debugging this main.c file and click on switch to change the perspective to debugger.

- Click on SWV ITM Data Console setting, enable the PC sampling and select port 0.

- Click on Start Trace (Red Dot) on SWV ITM Data Console tab and then click on resume button to start executing the main program. Now you can see the message on Port 0 and LED Toggle on STM32F446RE development board.

OBSERVATION:

Observation: During the experiment, the onboard LED (PA5) on the STM32F446RE board blinked regularly with an observed period of 500 ms, confirming that the FreeRTOS Task_1 executed successfully with osDelay(500) providing precise 500 ms blocking intervals. Simultaneously, the SWV ITM Data Console on Port 0 displayed the message "Task_1 Executing for LED Toggle" 2 times per second, perfectly synchronized with each LED toggle event, demonstrating successful ITM retargeting via the write () function. The program compiled with 0 errors and 0 warnings, and the debugger configuration maintained stable SWV tracing throughout the 2-3 minute observation period without any timing drift or missed task executions, validating both the CMSIS-RTOS v2 task creation and the non-intrusive debugging setup for Cortex-M4 embedded systems.

RESULT

A single task (Task_1) was successfully created using FreeRTOS Middleware software pack using CMSIS V2 interface and output is observed using SWV ITM Data console Port 0 on STM Cube IDE.

Reflection Summary

- How does using osDelay(500) inside Task1_function differ conceptually from implementing a 500 ms LED delay using a super-loop with HAL_Delay(500) or tick-based counters?

Ans- Using osDelay(500) inside a task is non-blocking for the system. It puts only that task to sleep for 500 ms and lets the RTOS run other tasks during that time. So the CPU is still used efficiently.

In a super-loop with HAL_Delay(500), the delay is blocking. The whole program stops for 500 ms, and nothing else can run during that time unless you manually manage it with counters.

In simple terms, osDelay() pauses just one task and shares CPU, while HAL_Delay() pauses everything and wastes CPU time.

- In this experiment only one task is running; what advantages does the RTOS-based approach provide now, and what additional benefits will appear when you add more tasks (e.g., UART logging, sensor reading)?

Ans- When only one task is running, RTOS doesn’t give a big performance advantage, but it still provides a clean structure. Your code is organized into tasks instead of one big loop, which makes it easier to read, debug, and modify. It also introduces you to how scheduling and delays work in a proper system.

The real benefits appear when you add more tasks. Then the RTOS can run multiple things at the same time—like LED control, UART logging, and sensor reading—without blocking each other. Each task runs independently, and the scheduler manages timing and switching automatically. This makes the system more efficient, scalable, and much easier to expand compared to a super-loop.

- What did you observe in the SWV ITM Data Console when the LED was blinking, and how does this help you verify correct task timing and execution without adding extra GPIO or UART prints?

Ans- When the LED was blinking, you would see regular messages appearing in the SWV ITM Data Console at fixed time intervals (around every 500 ms). The messages keep repeating continuously, which shows that the task is running again and again without stopping.

This helps you verify that the task timing is correct because the gap between messages matches the delay you set. It also proves the task is alive and executing properly. If the timing was wrong, you would see irregular or delayed messages. If the task stopped, the messages would stop too.

In simple words, the console acts like a live window into your code—you can confirm correct timing and execution without using extra pins or UART.

- If the LED did not blink or no messages appeared in the SWV console, which configuration steps in the procedure (GPIO, FreeRTOS, debugger/SWV settings, _write retargeting) would you systematically re-check first, and why?

Ans- If the LED is not blinking and no messages appear in the SWV console, you should first check the GPIO configuration to make sure the correct pin is set as output, because if this is wrong the LED will never turn on. Then check the FreeRTOS setup to confirm that the task is created properly and the scheduler is started, otherwise the task will not run at all. After that, verify the task code to ensure it is not stuck and that osDelay() is used correctly. Next, check the SWV/debugger settings like enabling SWV .